
[image: image14.jpg]LAING O'ROURKE




The Elevated Work Platform Minimum Standards are structured as follows:

	MINIMUM REQUIREMENTS
	Required for all elevated work platforms irrespective of the owner (LORA, Hire Company, Supply Chain Partner)

	LORA PROCUREMENT REQUIREMENTS
	Required for all elevated work platforms procured by LORA

	ADDITIONAL REQUIREMENTS BY TYPE
	Required for all plant of that type (i.e. boom lifts)

	MINIMUM DOCUMENT REQUIREMENTS
	Documentation required for all elevated work platforms

	PROJECT/SITE SPECIFIC REQUIREMENTS
	Project/site specific requirements for elevated work platforms as determined by the project risk assessment


	FEATURE
	REQUIREMENTS

	MINIMUM REQUIREMENTS – ELEVATED WORK PLATFORMS

	DESIGN COMPLIANCE PLATE TO

AS1418.10 FITTED
	Section 4.2.1 Manufacturer’s plate

One or more durable plate(s) giving the following information, which shall be indelibly marked and permanently attached to the MEWP in an easily visible place:

a) Manufacturer’s or supplier’s name.

b) Country of manufacture.

c) Model designation.

d) Serial number on a fixed component on the MEWP.

e) Year of manufacture.

f) Unloaded mass, in kilograms.

g) For insulated line vehicle MEWPs, the additional mass of tools and equipment required to be stored on the vehicle to satisfy the enhanced stability requirements (seeAS1418.10 Clause 7.8.2).

h) Rated capacity, in kilograms.

i) Rated capacity, given as the allowable number of persons and mass of equipment, in kilograms.

j) Maximum allowable manual force, in Newtons.

k) Maximum allowable wind speed, in metres per second or, where the MEWP is designed for use in non-wind conditions, advice to that effect.

l) Maximum allowable chassis inclination.

m) Hydraulic supply information, if an external hydraulic power supply is used.

n) Pneumatic supply information, if an external pneumatic power supply is used.

o) Electrical supply information, if an external electric power supply is used.

p) Installer of a vehicle-mounted MEWPs name (if applicable).

q) For insulated MEWPs, the electrical insulation rating (see AS1418.10 Section 7).

r) Statement of compliance with this Standard.

s) The rated capacity including platform capacity and lifting attachment capacity (if applicable).

	FIRE EXTINGUISHER + BRACKET
	The equipment shall be equipped with a portable dry chemical (ABE) fire extinguisher containing a minimum 1.5 kg of extinguishing agent (for boom type and diesel powered EWP’s a minimum 4.5kg of extinguishing agent)

The extinguisher/s should be:

· Fitted in an easily accessible and secure location

· Fitted with legible instructions and coloured correctly (i.e. red with white band)

· Fitted with a pressure gauge and hose to assist accessing awkward areas

· Maintained in accordance with AS 1851

NOTE: This is a minimum requirement. Specific requirements for extinguisher type and/or capacity may be defined by certain work activities or sites

	FLASHING AMBER BEACON (360 DEGREES)
	At least one amber beacon is fitted, with easily accessible on/off toggle switch to operate. The beacon shall be mounted as near as possible to the top of the equipment and be clearly visible in normal daylight up to a distance of 200 metres (and closing) in all directions.


[image: image1]© Laing O’Rourke, all rights reserved.
1

Published Date 19/07/2023

	FEATURE
	REQUIREMENTS

	REVERSE / TRAVEL / SLEWING ALARM (BEEPING NOISE ONLY)
	Fitted to central, unobstructed area of equipment. The alarm’s noise level range shall be 87 to 112 dB(a) measured at a distance of 1 metre from the alarm. The alarm shall active each time equipment movement is present (dependent on requirement and feature of alarm fitted i.e. reversing alarm) and not be fitted with an on/off feature.

	LOCKABLE BATTERY ISOLATION SWITCH C/W IDENTIFICATION DECAL
	A means of immobilising the equipment by isolating the battery so the equipment cannot be started and/or operated by an unauthorised person. The isolation switch must be lockable, resistant to weather and clearly identified (i.e. isolator decal).

Required Colour:
Battery Isolator – Red

	EMERGENCY STOP/S (WITH DECAL) EASILY AND SAFELY ACCESSIBLE FROM OUTSIDE PLANT
	Shall comply with AS1418.10, section 2.6.6 Emergency Stops

MEWPs shall be provided with emergency stop controls according to AS 4024.1604 or ISO 13850 at each control position.

All emergency stop actuators at all control locations other than at the platform (see Clause 2.6.5) shall remain active in all control modes. They shall be of the normally closed, positive-break type.

NOTE: For MEWPs that employ mechanically activated gravity lowering control(s), the lowering control may remain active after initiation of the emergency stop.

	GUARDING (ENGINE, ROTARY PARTS, HEAT SHIELDS, SOUND CLADDING)
	Guarding is in place, mitigating operator exposure to moving and/or hot parts and conform to National Standard for Plant NOHSC: 1010 (1994), clause 7, 8, 9 & 10 and, in addition, Select Plant Hire Equipment Safe Guarding Standard.

	SAFETY SIGNAGE IN PLACE FOR PINCH POINTS,

WARNINGS, EMERGENCY, POWER LINES, ETC.
	Shall comply with AS1418.10, section 4.2 Marking and symbols comply with ISO 20381. (i.e. safe working load (SWL) of the basket, safety around powerlines, etc.)

	SAFE ACCESS ONTO MACHINES (THREE POINTS OF CONTACT)
	Shall comply with AS1418.10, section 2.5.4 Guardrail (Protection) Systems and section 2.5.6 Openings in Guardrails for Entrance and Exit.

	BASKET/PLATFORM WITH HAND RAILS ON ALL SIDES
	Shall comply with AS1418.10, section 2.5.4 Guardrail (Protection) Systems.

Hand railing shall be minimum 950mm high and mid rails no further than 550mm from both the top hand rail and kick plate.

	KICK PLATES AROUND ALL EDGES OF BASKET/PLATFORM
	Shall comply with AS1418.10, section 2.5.4 Guardrail (Protection) Systems – minimum kick plate size of 100mm wide.

	DESIGNATED PERSONAL FALL ARREST EQUIPMENT ANCHOR POINTS (UNDAMAGED, LABELLED)
	Shall comply with AS1418.10, section 4.2.2 Work Platform, anchor points shall be labelled and legible and section 2.5.5 Anchorage(s).

	GATE OR DROP BAR SELF CLOSES AND HELD BY LATCH, SPRING OR GRAVITY
	Shall comply with AS1418.10, section 2.5.6 Openings in Guardrails for Entrance and Exit. Gates should be self-closing/opening and not secured by a pin or other means.

	SAFE WORKING LOAD (SWL) MARKED CLEARLY ON PLATFORM AND COMPLIANCE PLATE
	Shall comply with AS1418.10, sections:

· 4.2.1 Manufacturer’s Plate, sub section (h) and (i)

· 4.2.2 Work Platform, sub section (a) and (b)

· 4.2.5 Work Platform Rated Capacities

	MAXIMUM ALLOWABLE SIDE FORCE CLEARLY MARKED ON PLATFORM
	Shall comply with AS1418.10, section 4.2.2 Work Platform, Sub section (c) “The maximum allowable manual force, in Newtons”.

NOTE: Side force shall be a minimum of 200N for 1 person or 400N for more than 1 person rating.

	MAXIMUM ALLOWABLE CHASSIS INCLINATION (SIDE SLOPE) CLEARLY MARKED ON COMPLIANCE PLATE
	Shall comply with AS1418.10, section 4.2.1 Manufacturer’s Plate, sub section (l) “Maximum allowable chassis inclination”.


	FEATURE
	REQUIREMENTS

	WIND RATING MARKED ON PLATFORM, INCLUDES INDOOR AND OUTDOOR USE
	Shall comply with AS1418.10, section 4.2.2 Work Platform, sub section (d) “Maximum allowable wind speed, in metres per second or where the MEWP is designed for use in non-wind conditions, advice to that effect”.

NOTE: Outdoor use SHALL be at least 12.5m/s

	ALL CONTROLS/INSTRUMENTS (PLATFORM & GROUND) CLEARLY LABELLED AND

LEGIBLE
	Shall comply with AS1418.10, section 4.2.2 Work Platform all controls, anchor points, etc. shall be labelled and legible.

	DEAD-MAN FUNCTION INTEGRAL TO CONTROLS
	Shall be OEM designed and installed, compliant with AS1418.10. If the dead-man function is not activated the operator should not be able to activate any hand control to move the MEWP.

	ELECTRICAL HAZARD ‘NO-GO ZONE’ DECAL FITTED AND LEGIBLE (DISTANCE AS PER STATE REGS.)
	Shall comply with AS1418.10, section 4.2.2 Work Platform, sub section (g) “The safe approach distances to overhead powerlines”.

NOTE: “NO-GO ZONE” symbol/s shall comply with ISO 20381.

	BASKET CONTROL LEVERS PROTECTED FROM UNINTENDED OPERATION (EG.

GUARD)
	Shall comply with AS1418.10, section 2.6.2 Activation and Operation

“Control devices that control any movement of the MEWP shall automatically return to the ‘off’ or ‘neutral’ position when released”.

	RATED LIFTING POINTS C/W IDENTIFICATION DECAL
	OEM approved rated lifting point or forklift tyne access c/w decals. The lifting point must form a closed eye or similar form so that accidental unhooking of the

equipment cannot occur.

	ADDITIONAL LORA PROCUREMENT REQUIREMENTS – ELEVATED WORK PLATFORMS

	ASSET IDENTIFICATION STICKERS

(SELECT OWNED ASSETS ONLY)
	Asset Decals: Black lettering on a white background, non-reflective adhesive backed decal

Suggested Size: 200 x 100mm

Installation: Evenly applied to LH and RH side of asset

	ADDITIONAL REQUIREMENTS FOR BOOM LIFTS

	SECONDARY GUARDING FOR TELESCOPIC BOOM TYPE (REFER TO MANUFACTURER FOR MORE INFORMATION)
	
[image: image2]

[image: image3]
Operator Protection Alarm (OPA)
Sky Guard

	SECONDARY GUARDING FOR SCISSOR LIFTS(REFER TO MANUFACTURER FOR MORE INFORMATION
	
[image: image4][image: image5.jpg]



Operator Prescence System (e.g. EQSS)

	DOCUMENTATION MINIMUM REQUIREMENTS

(Paper or electronic – Must be available at the time of mobilisation)

	OPERATORS MANUAL
	OEM operators manual available and in good condition.

	PLANT RISK ASSESSMENT
	Specific to make and model of equipment.

	DAILY PRE-START BOOK
	Specific to category of equipment of equipment.

	SERVICE HISTORY
	Previous 3 months of preventative maintenance and repairs.

	EWP LOGBOOK
	Placed in EWP document holder.

	PLANT REGISTRATION
	Worksafe Registration, relevant to equipment and state regulations.

	CHANGE MANAGEMENT
	Change management documents available for any modifications

	QUARTERLY INSPECTION RECORD
	Placed in EWP document holder, relevant to specific plant and completed by a competent person.

	ANNUAL INSPECTION RECORD
	Placed in EWP document holder, relevant to specific plant and completed by a competent person.


	FEATURE
	REQUIREMENTS

	PROJECT / SITE / CLIENT SPECIFIC REQUIREMENTS

Items that may be required as determined by the project/site and/or client

	WHEEL NUT INDICATORS
	Fitted to wheel nuts to help identify any loose nuts, preventing the possibility of wheel detachment. Indicators should be of a suitable design and yellow in colour.

	WEED AND SEED
	A statutory declaration for all Plant and Equipment that they are clean and free from weed and seeds. This is to confirm that the Plant and Equipment has not gone through areas with weed and in particular (parthenium weed or giant rat's-tail grass). If the item has gone through areas of this weed, a clean down certificate will be required.

	CALL SIGNS

(HIGH VIS OR STANDARD)
	Used for communication purposes between road users.

Call Sign Decals: Typically, black lettering on a reflective yellow high vis background (high vis version) or black lettering on silver or white non-reflective background (standard, non-mine spec)

Suggested Size: 500 x 250mm

Installation: Evenly applied to LH, RH and rear of asset

	SPILL KIT
	General vehicle spill kit design to assist in small fluid (i.e. oil or coolant) spills. The spill kit bag shall be weather proof to avoid damaging contents.

	WHEEL CHOCKS WITH SECURE CARRYING MOUNTS (FOR RUBBER TYRED EQUIPMENT)
	Rubber and compatible with equipment size and requirements. Shall be secured to equipment with bracket to prevent unintended movement when not in use.

	SECONDARY GUARDING (REFER TO OEM FOR MORE INFORMATION)
	[image: image6.jpg]Operator Protective Structure (OPS)





	SPECIFIC TYRES (IF AVAILABLE FOR TYPE OF MACHINE/TASK)
	All Terrain (outdoor use)
Smooth (indoor use)

[image: image7.jpg]



[image: image8.jpg]




	REFLECTIVE TAPE
	Material: The tape shall be weather resistant with a reflective surface and adhesive back

Size: Minimum 50mm wide

Installation: One strip evenly applied to all sides of the equipment; in addition boom sections also (optional)

	EMERGENCY DECENT DEVICE (E.D.D.)
	Device used for emergency evacuations from EWP baskets or other work platforms.

	LOCKABLE STARTER ISOLATION SWITCH C/W IDENTIFICATION DECAL
	A means of immobilising the equipment starter. The isolation switch must be lockable, resistant to weather and clearly identified.

Required Colour: Starter Isolator – Yellow


ELEVATED WORK PLATFORM


MINIMUM STANDARDS





�











[image: image9.png]


[image: image10.png]


[image: image11.png]


[image: image12.jpg]


[image: image13.jpg]


