LIFTING CATEGORIES AND ASSOCIATED APPROVALS

| 1 | Load Details |  | Lift Analysis | Lift Study Plan | Critical Lift Plan (Note 5) |
| --- | --- | --- | --- | --- | --- |
| 1.1 | Gross load over 50Te and below 90% or greater of the rated Crane capacity |  |  | √ |  |
| 1.2 | Gross load over 90% or greater of the rated Crane capacity / Any Prohibited Lifts |  |  |  | √ |
| 1.3 | Gross load under 50Te and between 80% and 90% of the rated Crane capacity |  |  | √ |  |
| 1.4 | Gross load under 15Te and under 80% of the rated Crane capacity |  | √ |  |  |
| 2 | Crane Set-Up |  | Lift Analysis | Lift Study Plan | Critical Lift Plan (Note 5) |
| 2.1 | Assessment of ground to be completed by Temp Work Co-Ordinator, permit to Load must be approved before set-up of crane begins |  | √ | √ | √ |
| 2.2 | Crane on a suspended platform or over critical infrastructure or services |  |  | √ |  |
| 2.3 | Crane track or outrigger pad/s close to embankment or excavation |  |  | √ |  |
| 2.4 | Partially extended outriggers |  |  | √ |  |
| 2.5 | Special lift devices (e.g. bespoke gantry systems, climbing jacks, strand jacks, derricks) |  |  | √ |  |
| 2.6 | Land based cranes on barges (Note 1) |  |  |  | √ |
| 2.7 | Lifts over public roads and highways (all risks that could impact on major infrastructure must have appropriate controls in place including emergency response and contingency plans - post consultation with relevant authorities) |  |  | √ |  |
| 3 |  |  | Lift Analysis | Lift Study Plan | Critical Lift Plan (Note 5) |
| 3.1 | Lifting with single crane and two (2) winches |  |  | √ |  |
| 3.2 | Where possibility of encroaching overhead powerline safe approach distance exists |  |  | √ |  |
| 3.3 | Two (2) crane lifts involving complex manoeuvre and / or pass out operations |  |  | √ |  |
| 3.4 | Super lift operation |  |  | √ |  |
| 3.5 | Multiple Crane Lifts - two (2) or more cranes |  |  |  | √ |
| 3 | Crane Operation |  | Lift Analysis | Lift Study Plan | Critical Lift Plan (Note 5) |
| 3.6 | Congested areas |  | √ |  |  |
| 3.7 | Standard Crane Set-up |  | √ |  |  |
| 3.8 | Pick and Carry Operations by Cranes (including Pick and Carry operations undertaken by Forklift, Telehandler or Excavator) (Note 2) |  |  | √ |  |
| 3.9 | Dual Winch Lifting |  |  |  | √ |
| 4 | Rigging and Loading |  | Lift Analysis | Lift Study Plan | Critical Lift Plan (Note 5) |
| 4.1 | Custom designed / bespoke rigging |  |  | √ |  |
| 4.2 | Workboxes (lifting personal lifts involving persons suspended in Workbox) Note: must also meet all the criteria of the Workbox permit |  |  |  | √ |
| 4.3 | Precast Tilt up Panels |  |  | √ |  |
| 4.4 | Complex Rigging |  |  | √ |  |
| 4.5 | Pre-Cast elements (excluding tilt up) |  | √ |  |  |
| 4.6 | Demolition (including pile removal) (Note 3) |  |  |  | √ |
| 4.7 | Loads with no engineered lift points or lifting method is difficult to sling |  | √ |  |  |
| 4.8 | Loads with OEM approved lift points or slinging method |  | √ |  |  |
| 4.9 | Simple rigging arrangements (including spreader bars and lifting beams) |  | √ |  |  |
| 4.10 | Lifts involving submerged objects |  |  |  | √ |

| Notes |  |
| --- | --- |
| Note 1: Where land-based cranes are to be used on barges the crane must be de-rated accordingly and barge checked by Naval Architect for suitability. Dynamic Amplification Factors are to be applied to the gross lifted load in accordance with marine lifting standards. Alternatively, a Naval Architect must be engaged to determine accurate dynamic amplification factors based on specific conditions |
| Note 2: Dynamic effects on crane, rigging and load must be considered in all piling and pick & carry related activities. |
| Note 3: In all cases, Lift Studies / Plans must be reviewed and approved by the Project AP: |
| Note 4: Climbing Operations of Tower Cranes must be fully planned and approved in consultation with SELECT Cranes Lifting Solutions & Temporary Works Co-ordinator |
| Note 5: All Critical Lift Plans need to be approved by a Project Director / Leader or ref Project Specific Guidelines for approval, refer to PS Cranes and Lifting and the Appointed Person Support Library (LOR only). |